“The only limits to the possibilities in your life tomorrow

are the buts you use today”

“les brown”

Current Electricity

Electric Current :Current is the rate at which charge flows through a surface.

Iav =  ∆Q /∆t                                                                      (27.1)

The SI unit of current is the ampere (A):

1 A = 1C/1s                                              (27.3)

- It is conventional to assign to the current the same direction as the flow of positive charge.

Drift speed-The speed of the charge carriers vd is an average speed called the drift speed.

Iav =  ∆Q /∆t    = (nA vd )q =       nq vd A              (27.4)

Resistance and Ohm’s Law

A current density J and an electric field E are established in a conductor whenever a potential difference is maintained across the conductor.

If the potential difference is constant, then the current also is constant. In some materials, the current density is proportional to the electric field:

J  =  E                                                          (27.7)

where the constant of proportionality  is called the conductivity of the conductor.

Materials that obey Equation 27.7 are said to follow Ohm’s law, named after Georg Simon Ohm (1787–1854). More specifically, Ohm’s law states that

“for many materials (including most metals), the ratio of the current density to the electric field is a constant  that is independent of the electric field producing the current. ”

Materials that obey Ohm’s law and hence demonstrate this simple relationship between E and J are said to be ohmic. Experimentally, it is found that not all materials have this property, however, and materials that do not obey Ohm’s law are said to be nonohmic.

Ohm’s law is not a fundamental law of nature but rather an empirical relationship valid only for certain materials.

J= E we obtain the following relationships for conductivity and resistivity:

According to this classical model, conductivity and resistivity do not depend on the strength of the electric field. This feature is characteristic of a conductor obeying Ohm’s law.

Mobility  μ is defined as the magnitude of the drift velocity per unit electric field:

μ =| vd |/ E

The SI unit of mobility is m2/Vs . Mobility is positive.  Since vd = e τE/m

where e –charge on an electron , τ = average collision time for electrons and m is mass of an electron

Hence, μ =| vd |/ E  = e τ/m

Limitations Of Ohm’s Law

(a) V ceases to be proportional to I (.

(b) The relation between V and I depends on the sign of V. In other words, if I is the current for a certain V, then reversing the direction of V keeping its magnitude fixed, does not produce a current of the same magnitude as I in the opposite direction . This happens, for example, in a diode .

(c) The relation between V and I is not unique, i.e., there is more than one value of V for the same current I . A material exhibiting such behaviour is GaAs(Gallium arsenide (GaAs), are used as semiconductors. GaAs is also used as a laser material.)

Temperature Dependence Of  Resistivity             ρT = ρ0 [1 +  (T–T0)]

We can qualitatively understand the temperature dependence of resistivity, in the light of our derivation of Eq.

From this equation, resistivity of a material is given by

ρ thus depends inversely both on the number n of free electrons per unit

volume and on the average time τ between collisions. As we increase temperature, average speed of the electrons, which act as the carriers of current, increases resulting in more frequent collisions. The average time of collisions τ, thus decreases with temperature.

.

The resistors have a set of co-axial coloured rings on them whose significance are listed in Table 27.2.

The first two bands from the end indicate the first two significant figures of the resistance in ohms. The third band indicates the decimal multiplier (as listed in Table (27.2). The last band stands for tolerance or possible variation in percentage about the indicated values.

Sometimes, this last band is absent and that indicates a tolerance of 20% (Fig. 3.8). For example, if the four colours are orange, blue, yellow and gold, the resistance value is 36 × 104 Ω, with a tolerance value of 5%.

BB  Roy of Great Britain has very good wife wearing Gold and silver

Electrical Energy And Power

The rate at which the charge ∆Q loses potential energy in going through the resistor is

∆U/∆t =∆Q∆V/∆t   = I ∆V

we have  P = I ∆V                                  (27.22)

We can express the power delivered to the resistor in the alternative forms

P = I ∆V  = I2R   = P = ∆V2/R                                                 (27.23)

When I is expressed in amperes, ∆V in volts, and R in ohms, the SI unit of power is the watt.

The power lost as internal energy in a conductor of resistance R is called joule heating this transformation is also often referred to as an I 2R loss.

P = I 2 R = V 2/R                                                                                          (27.23A)

Quiz : When does a light bulb carry more current—just after it is turned on and the glow of the metal filament is increasing, or after it has been on for a few milliseconds and the glow is steady?

Solution : Just after it is turned on. When the filament is at room temperature, its resistance is low, and hence the current is relatively large As the filament warms up, its resistance increases, and the current decreases. Older lightbulbs often fail just as they are turned on because this large initial current “spike” produces rapid temperature increase and stress on the filament

Quiz : The same potential difference is applied to the two light bulbs of 30W and 60W  Which one of the following statements is true?

(a) The 30-W bulb carries the greater current and has the higher resistance.

(b) The 30-W bulb carries the greater current, but the 60-W bulb has the higher resistance.

(c) The 30-W bulb has the higher resistance, but the 60-W bulb carries the greater current.

(d) The 60-W bulb carries the greater current and has the higher resistance.

Solution : (c). Because the potential difference ∆V is the same across the two bulbs and because the power delivered to a conductor is P=I∆V,the 60-W bulb, with its higher power rating, must carry the greater current. The 30-W

bulb has the higher resistance because it draws less current at the same potential difference.

Quiz : For the two lightbulbs shown in Figure 27.17, rank the current values at points a through f , from greatest to least.

Solution : Ia = Ib > Ic = Id > Ie = If .

The current Ia leaves the positive terminal of the battery and then splits to flow through the two bulbs; thus,

Ia = Ic + Ie

From previous Quiz, we know that the current in the 60-W bulb is greater than that in the 30-W bulb. (Note that all the current does not follow the “path of least resistance,” which in this case is through the 60-W bulb.)

Because  charge does not build up in the bulbs, we know that all the charge flowing into a bulb from the left must flow out on the right; consequently,

Ic = Id    and Ie = If

The two currents leaving the bulbs recombine to form the current back into the battery,      If + Id  =  Ib

_________________________________

ELECTROMOTIVE FORCE        ε = IR +Ir

The potential difference across the load resistance isV = IR

Solving for the current gives           I = ε / ( R+r

_______________________________________________________________________

Problem : Terminal Voltage of a Battery

A battery has an emf of 12.0 V and an internal resistance of 0.05 ohm. Its terminals are connected to a load resistance of 3.00 ohm.

(a) Find the current in the circuit and the terminal voltage of the battery

(b) Calculate the power delivered to the load resistor, the power delivered to the internal resistance of the battery, and the power delivered by the battery.

Solution Using first Equation 28.3 and then Equation 28.1, we obtain

I = ε / ( R+r )  = 12/3.05 =3.93 A

∆V = Vb - Va = ε – Ir = 12 – (3.93 x 0.05) = 11.8V

To check this result, we can calculate the voltage across the load resistance R:

∆V  = IR =  (3.93 )(3.00 ) = 11.8 V

(b) The power delivered to the load resistor is  P =I 2R = (3.93)2 x 3 = 46.3W

The power delivered to the internal resistor is  P = I 2r = (3.93)2 x 0.05 = 0.772W

Hence, the power delivered by the battery is the sum of these quantities, or 47.1 W. You should check this result, using the expression P = I ε.

Resistors In Series                               Req     = R1 + R2 + R3  +R4+...... (28.6)

Resistors In Parallel

Quiz : Are automobile headlights wired in series or in parallel? How can you tell?

Solution : They must be in parallel because if one burns out, the other  continues to operate. If they were in series, one failed headlamp would interrupt the current throughout the entire circuit, including the other headlamp.

Problem : Find the Equivalent Resistance

Four resistors are connected as shown in Figure 28.6a

(a) Find the equivalent resistance between points a and c.

(b) What is the current in each resistor if a potential difference of 42V is maintained between  a and c ?

Solution The combination of resistors can be reduced in steps, as shown in Figure 28.6.

The 8.0-Ω and 4.0- Ω resistors are in series; thus, the equivalent resistance between a and b is 12 Ω (see Eq. 28.5).

The 6.0- Ω and 3.0- Ω resistors are in parallel, so from Equation 28.7 we find that the equivalent resistance from b to c is 2.0 Ω. Hence, the equivalent resistance from a to c is 14 Ω

(b) The currents in the 8.0- Ω and 4.0- Ω resistors are the same because they are in series. In addition, this is the same as the current that would exist in the 14- Ω equivalent resistor subject to the 42-V potential difference.

Therefore, using Equation 27.8 and the results from part (a), we obtain

I =  ∆Vac /Req = 42/14 = 3 A

This is the current in the 8.0- Ω and 4.0- Ω resistors. When this 3.0-A current enters the junction at b , however, it splits, with part passing through the 6.0- Ω resistor (I1) and part through the 3.0- Ω resistor (I2).

Because the potential difference is ∆Vbc across each of these resistors (since they are in parallel),

we see that                   (6.0 Ω) I1 = (3.0 Ω ) I2                 or I2 = 2I1

Using this result and the fact that I1 + I 2 = 3.0 A, we find that I1 = 1.0 A and

I2 = 2.0 A

We could have guessed this at the start by noting that the current through the 3.0- Ω resistor has to be twice that through the 6.0- Ω resistor, in view of their relative resistances and the fact that the same voltage is applied to each of them.

As a final check of our results note that ∆Vbc = (6)I1 = (3)I2 = 6V and ∆Vab = (12) I =36 V ,  therefore  ∆Vac =  ∆Vab  +     ∆Vbc  = 42 V

Problem : Three Resistors in Parallel

Three resistors are connected in parallel as shown in Figure 28.7. A potential difference of 18 V is maintained between points a and b.

(a) Find the current in each resistor.

(b) Calculate the power delivered to each resistor and the total power delivered to the combination of resistors.

(c) Calculate the equivalent resistance of the circuit.

Solution The resistors are in parallel, and so the potential difference across each must be 18 V. Applying the relationship ∆V =IR to each resistor gives

I1 = ∆V/R1 =  18/3 = 6 A

I2 = ∆V/R2 =  18/6 = 3 A

I3 = ∆V/R3 =  18/9 = 2 A

(b) We apply the relationship P =V2/R to each resistor and obtain

P1 = ∆V2/R1 =  (18)2/3 = 110W     	P2 = ∆V2/R2 =  (18)2/6 = 54W

P3 = ∆V2/R3 =  (18)2/9 = 36W

This shows that the smallest resistor receives the most power.

Summing the three quantities gives a total power of 200 W.

(c) We can use Equation 28.8 to find Req :

1/Req = 1/R1 + 1/R2 + 1/R3  = 1/3 +1/6 +1/9 = 11/18

Or Req = 18/11 = 1.6 ohm

Exercise Use Req to calculate the total power delivered by the battery

Answer       P =V2/Req = (18)2/1.6 =  200 W.

________________________________________________________________________

Problem : Finding Req by Symmetry Arguments

Consider five resistors connected as shown in Figure 28.8a.Find the equivalent resistance between points a and b.

Solution In this type of problem, it is convenient to assume a current entering junction a and then apply symmetry arguments.

Because of the symmetry in the circuit (all 1ohm resistors in the outside loop), the currents in branches ac and ad must be equal; hence, the electric potentials at points c and d must be equal. This means that  ∆Vcd = 0 and, as a result,points c and d may be connected together without affecting the circuit, as in Figure 28.8b.

Thus, the 5ohm resistor may be removed from the circuit and the remaining circuit then reduced as in Figures 28.8c and d. From this reduction we see that the equivalent resistance of the combination is 1 ohm. Note that the result is 1 ohm regardless of the value of the resistor connected between c and d.

Cells In Series     εeq = ε1 + ε2   and     req = r1 + r2

Cells In Parallel

Kirchhoff’s Rules

1. Junction rule : The sum of the currents entering any junction in a circuit must equal the sum of the currents leaving that junction:

I in =  Iout                                                         (28.9)

2. Loop rule: The sum of the potential differences across all elements around any closed circuit loop must be zero:

∆V = 0        (closed loop)                                                             (28.10)

The Ammeter

A device that measures current is called an ammeter. The current to be measured must pass directly through the ammeter, so the ammeter must be connected in series with other elements in the circuit, as shown in Figure 28.21. When using an ammeter to measure direct currents, you must be sure to connect it so that current enters the instrument at the positive terminal and exits at the negative terminal.

Ideally, an ammeter should have zero resistance so that the current being measured is not altered. In the circuit shown in Figure 28.21, this condition requires that the resistance of the ammeter be much less than(R1+R2) Because any ammeter always has some internal resistance, the presence of the ammeter in the circuit slightly reduces the current from the value it would have in the meter’s absence.

The Voltmeter

A device that measures potential difference is called a voltmeter. The potential difference between any two points in a circuit can be measured by attaching the terminals of the voltmeter between these points without breaking the circuit, as shown in Figure 28.22. The potential difference across resistor R2 is measured by connecting the voltmeter in parallel with R2 .

Again, it is necessary to observe the polarity of the instrument. The positive terminal of the voltmeter must be connected to the end of the resistor that is at the higher potential, and the negative terminal to the end of the resistor at the lower potential.

An ideal voltmeter has infinite resistance so that no current passes through it. In Figure 28.22, this condition requires that the voltmeter have a resistance much greater than R2 . In practice, if this condition is not met, corrections should be made for the known resistance of the voltmeter.

The Wheatstone Bridge

An unknown resistance value can be accurately measured using a circuit known as a Wheatstone bridge (Fig. 28.25). This circuit consists of the unknown resistance Rx , three known resistances R1 , R2 , and R3 (where R1 is a calibrated variable resistor),a galvanometer, and a battery.

The known resistor R1 is varied until the galvanometer reading is zero—that is, until there is no current from a to b. Under this condition the bridge is said to be balanced. Because the electric potential at point a must equal the potential at point b when the bridge is balanced, the potential difference across R1 must equal the potential difference across R2 .

Likewise,the potential difference across R3 must equal the potential difference across Rx .

From these considerations we see that

(1) I1R1  =  I2R2                        (2)   I1R3  =  I2Rx

Dividing Equation (1) by Equation (2) eliminates the currents, and solving for Rx , we find that

Rx = R2 R3/ R1        (28.19)

A number of similar devices also operate on the principle of null measurement (that is, adjustment of one circuit element to make the galvanometer read zero). One example is the capacitance bridge used to measure unknown capacitances. These devices do not require calibrated meters and can be used with any voltage source.

A practical device using this principle is called the meter bridge.

Wheatstone bridges are not useful for resistances above 105 ohm, but modern electronic instruments can measure resistances as high as 1012 ohm for e.g. digital multimeters.

The Potentiometer

A potentiometer is a circuit that is used to measure an unknown emf εx by comparison with a known emf. In Figure 28.27, point d represents a sliding contact that is used to vary the resistance (and hence the potential difference) between points a and d. The other required components are a galvanometer, a battery of  known emf ε0 , and a battery of unknown emf εx .

With the currents in the directions shown in Figure 28.27, we see from Kirchhoff’s junction rule that the current in the resistor Rx is I -Ix where I is the current in the left branch (through the battery of emf ε0) and Ix is the current in the right branch.

Kirchhoff’s loop rule applied to loop abcda traversed clockwise gives

- εx + (I -Ix) Rx = 0

Because current Ix passes through it, the galvanometer displays a nonzero reading.

The sliding contact at d is now adjusted until the galvanometer reads zero (indicating a balanced circuit and that the potentiometer is another null-measurement device).

Under this condition, the current in the galvanometer is zero, and the potential difference between a and d must equal the unknown emf εx :

εx = I Rx

Next, the battery of unknown emf is replaced by a standard battery of known emf εs , and the procedure is repeated. If Rs is the resistance between a and d when balance is achieved this time, then

εs = I Rs

where it is assumed that I remains the same. Combining this expression with the preceding one, we see that

εx = εs Rx / Rs 	                       (28.20)

If the resistor is a wire of resistivity ρ ,its resistance can be varied by using the sliding contact to vary the length L, indicating how much of the wire is part of the circuit.

With the substitutions Rs = ρ Ls/A  and Rx = ρ Lx/A  Equation 28.20 becomes

εx = εs Lx / Ls                                      (28.21)

where Lx is the resistor length when the battery of unknown emf εx is in the circuit and Ls is the resistor length when the standard battery is in the circuit.

The sliding-wire circuit of Figure 28.27 without the unknown emf and the

galvanometer is sometimes called a voltage divider. This circuit makes it possible to tap into any desired smaller portion of the emf ε0 by adjusting the length of the resistor.

Meter Bridge (An Instrument used to measure unknown Resistance)

The meter bridge is shown in Fig. 3.27. It  consists of a wire of length 1m and of uniform cross sectional area stretched taut and clamped between two thick metallic strips bent at right angles, as shown. The metallic strip has two gaps across which resistors can be connected. The end points where the wire is clamped are connected to a cell through a key. One end of a galvanometer is connected to the metallic strip midway between the two gaps. The other end of the galvanometer is connected to a ‘jockey’.

The jockey is essentially a metallic rod whose one end has a knife-edge which can slide over the wire to make electrical connection.

R is an unknown resistance whose value we want to determine. It is connected across one of the gaps. Across the other gap, we connect a standard known resistance S. The jockey is connected to some point D on the wire, a distance l cm from the end A. The jockey can be moved along the wire. The portion AD of the wire has a resistance Rcml, where Rcm is the resistance of the wire per unit centimetre. The portion DC of the wire similarly has a resistance Rcm (100-l ).

The four arms AB, BC, DA and CD [with resistances R, S, Rcm l and

Rcm(100-l)] obviously form a Wheatstone bridge with AC as the battery arm and BD the galvanometer arm. If the jockey is moved along the wire,then there will be one position where the galvanometer will show no current.

Let the distance of the jockey from the end A at the balance point be l= l1.

The four resistances of the bridge at the balance point then are R, S, Rcm l1 and Rcm(100–l1). The balance condition, gives

Thus, once we have found out l1, the unknown resistance R is known in terms of the standard known resistance S by

By choosing various values of S, we would get various values of l1,and calculate R each time. An error in measurement of l1 would naturally result in an error in R. It can be shown that the percentage error in R can be minimised by adjusting the balance point near the middle of the bridge, i.e., when l1 is close to 50 cm. (This requires a suitable choice of S.)

________________________________________________________________

Some points :

Maximum Power Theorem :

In an electrical circuit, the maximum power can be drawn from the battery when external resistance is same as the internal resistance of the battery.

Fuse wire :

Is a wire element made of a material with low resistivity and low melting point (mostly made of an alloy of tin and led) with suitable radius of cross section, connected in the series of circuit. The fuse wire blows out when current exceeding the limit to which the wire is made protecting the electrical appliances against the current surge.

-A conducting device obeys Ohm's law when the resistance of the device is independent of the magnitude and polarity of the applied potential difference.

-A conducting material obeys Ohm's law when the resistivity of the material is independent of the magnitude and direction of the applied electric field.

-Resistance Rule: For a move through a resistance in the direction of the current the change in potential is –iR ; in the opposite direction it is +iR.

-EMF RULE: For a move through an ideal emf device in the direction of the emf arrow,the change in potential is +ε ; in the opposite direction it is -ε.

Summary and shortcuts

-The electric current I in a conductor is defined as             I = dq/dt                  (27.2)

where dq is the charge that passes through a cross-section of the conductor in a time dt. The SI unit of current is the ampere (A), where 1 A= 1 C/s.

-The average current in a conductor is related to the motion of the charge carriers through the relationship

Iav = nqvdA                                          (27.4)

where n is the density of charge carriers, q is the charge on each carrier, vd is the drift speed, and A is the cross-sectional area of the conductor.

-The magnitude of the current density J in a conductor is the current per unit area:

J= I/A = nqvd                                               (27.5)

-The current density in a conductor is proportional to the electric field according to the expression

J= E                                                    (27.7)

The proportionality constant  is called the conductivity of the material of which the conductor is made. The inverse of   is known as resistivity ρ (ρ = 1/). Equation 27.7 is known as Ohm’s law, and a material is said to obey this law if the ratio of its current density J to its applied electric field E is a constant that is independent of the applied field.

- The resistance R of a conductor is defined either in terms of the length of the conductor or in terms of the potential difference across it:

R = ρ l/A= V/I                      (27.8)

If the resistance is independent of the applied potential difference, the conductor obeys Ohm’s law.

-In a classical model of electrical conduction in metals, the electrons are treated as molecules of a gas. In the absence of an electric field, the average velocity of the electrons is zero. When an electric field is applied, the electrons move (on the average) with a drift velocity vd that is opposite the electric field and given by the expression

Vd  =  qEτ/me                                (27.14)

-According to this model, the resistivity of the metal is        ρ = me/nq2τ      (27.17)

where n is the number of free electrons per unit volume.

-The resistivity of a conductor varies approximately linearly with temperature according to the expression

ρ  = ρ0 [ 1+ (T-T0)]            (27.19)

where  is the temperature coefficient of resistivity and ρ0 is the resistivity at some reference temperature T0.

-If a potential difference ∆V is maintained across a resistor, the power, or rate at which energy is supplied to the resistor, is

P = VI   = I2R = V2/R                            (27.22)

- The emf of a battery is equal to the voltage across its terminals when the current is zero. That is, the emf is equivalent to the open-circuit voltage of the battery.

- The equivalent resistance of a set of resistors connected in series is

Req  =  R1 + R2 + R3+.......            (28.6)

-The equivalent resistance of a set of resistors connected in parallel is

1/Req  = 1/R1 + 1/R2 + 1/R3                (28.8)

-Circuits involving more than one loop are conveniently analyzed with the use of Kirchhoff’s rules:

1. The sum of the currents entering any junction in an electric circuit must equal

the sum of the currents leaving that junction:

Iin =  Iout                                    (28.9)

2. The sum of the potential differences across all elements around any circuit loop must be zero:

∆V (closed loop) = 0                                  (28.10)

The first rule is a statement of conservation of charge; the second is equivalent to

a statement of conservation of energy.

-When a resistor is traversed in the direction of the current, the change in potential

∆V across the resistor is - IR.

- When a resistor is traversed in the direction opposite the current, ∆V = +IR

- When a source of emf is traversed in the direction of the emf (negative terminal to positive terminal), the change in potential is +ε.

-When a source of emf is traversed opposite the emf (positive to negative), the change in potential is - ε.

The use of these rules together with Equations 28.9 and 28.10 allows you to analyze electric circuits.

-To maintain a steady current, we must have a closed circuit in which an external agency moves electric charge from lower to higher potential energy. The work done per unit charge by the source in taking the charge from lower to higher potential energy (i.e., from one terminal of the source to the other) is called the electromotive force, or emf, of the source. Note that the emf is not a force; it is the voltage difference between the two terminals of a source in open circuit.

-In most substances, the carriers of current are electrons; in some cases, for example, ionic crystals and electrolytic liquids, positive and negative ions carry the electric current.

-Ohm’s law is obeyed by many substances, but it is not a fundamental law of nature. It fails if

(a) V depends on I non-linearly.

(b) the relation between V and I depends on the sign of V for the same absolute value of V.

(c) The relation between V and I is non-unique.

An example of (a) is when ρ increases with I (even if temperature is kept fixed). A rectifier combines features (a) and (b). GaAs shows the feature (c).

-When a source of emf ε is connected to an external resistance R, the voltage Vext across R is given by

Vext = IR  = εR / (R+r) ,

where r is the internal resistance of the source

- The Wheatstone bridge is an arrangement of four resistances using which the value of one resistance can be determined, knowing the other three resistances.

-The potentiometer is a device to compare potential differences. Since the method involves a condition of no current flow, the device can be used to measure potential difference; internal resistance of a cell and compare emf’s of two sources.

-If charge q is in Coulomb and t is in seconds then I is in ampere. lf q is in ab-Coulomb and t is in seconds,

then I is in ab-ampere or CGS e.m. unit of current.(1 ampere = 0.1 emu of current).

-Unit of resistance in MKS system is ohm while in CGS e.m. system is ab-ohm. One ohm = 109 ab-ohm

-Some other important points concerning variation of resistance:

(i)Resistance of bismuth wire increases when placed in a magnetic field.

(ii)Resistance of carbon granules decreases when pressure is increased.

(iii) Resistance of semiconductor decreases when light falls on it

(iv) Resistance of an intrinsic semiconductor decreases when doped with a trivalent or penta valent impurity.

(v) The specific resistance of alloys increases with a rise in temperature but this increase is much smaller when compared to Pure metals.

Electric cell:

-When a cell is being charged, then V = E + Ir, and when the cell is being discharged, then V = E - Ir

(where .I represents the current and r the internal resistance of the cell).

- In an open circuit, i.e., when I = 0 V = E.

- The internal resistance r of the cell is the resistance of the column of liquid between the two plates of the

cell.

- The internal resistance of the cell depends upon the (a) separation between the plates, (b) area of cross-section of the column of liquid between the plates and (c) nature, temperature and the degree of dissociation of electrolyte between them.

-Cells are of two types: (a) primary cells and (b) secondary cells. Voltaic cell, Leclanche cell, Daniel cell, etc., are primary cells while lead accumulators and alkali accumulators are secondary cells. (x) Advantages of secondary cells over primary cells are: (a) higher emf G) higher capacity, (c) more steady current, (d) smaller internal resistance, (e) can be recharged as reaction is reversible. (xi) Disadvantages of secondary cells over primary cells are: (a) larger weight, (b) more costly, (c) smaller self life, (d) get spoiled if short-circuited.

Combinations of cells:

(i) Two cells of emf E1 and E2 and internal resistances r1 and r2, when connected in series correctly, then equivalent emf E = E1 + E2 and equivalent internal resistance r = r1+ r2. However, if the two cells are connected wrongly then equivalent emf, E = E1- E2 but the equivalent internal resistance will remain same, i.e., r = r1+ r2.

(ii) If n cells, each of emf E and internal resistance r are connected in series with external resistance R, then

current

I = nE/(R+nr)         (a) I=Imax =nE/R, when R>>nr                         (b) I=Imin =nE/nr =E/r , when nr >> R

(iii) If m cells, each of emf E and internal resistance r are connected in parallel with each other and the combination is connected in series with an external resistance R, then

I = mE/(mR+r)

(iv) If n cells, each of emf E and internal resistance r are connected in series and m such rows are connected in parallel and the combination is connected in series with an external resistance R, then

I = mnE/(mR+nr)

(v) (a) It is useful to connect cells in series, when R >> r.

(b) It is useful to connect cells in parallel, when R>>r.

(c) It is useful to connect cells in series-parallel mixed grouping when R=r. For maximum current,

R >> nr / m.

- Kirchhoff’s First law is based on the law of conservation of charge'

- Kirchhoff’s First law is based on the law of conservation of energy

Whetstone’s bridge:

(i) In Whetstone’s bridge, four resistances P, Q, R and S are connected such that P and Q are in series and so are R and S. and  P+Q branch is in parallel with R+S branch as shown in the following circuit diagram when key K2 is open. When the bridge is balanced,    QS=PR and no current flows in the arm BD.

(ii) When QS > PR; current flows from B to D.

(iii) When QS < PR, current flows from D to B.

(iv) The bridge is most sensitive when all the four resistors are of the same order.

(v) Metre bridge, post office box and Carey Foster's bridge are practical applications of Whetstone’s bridge.

Potentiometer:

(i) A potentiometer generally consists of ten wires of uniform area of cross-section stretched on a wooden board between two thick copper strips. Each wire is 100 cm long. The wire is usually, made of constantan or manganin. A metre scale is fitted parallel to its length

(ii) A steady current is set up in the wire by means of a battery B. This maintains a uniform potential gradient (fall of potential per unit length) along the length of the wire. In the Fig. 31.2, E is the emf of the source which is balanced between points P and C.

(iii)Let R be the total resistance of the potentiometer wire having a total length L. Then, potential gradient R =I x R/L

(iv) Any potential difference less than the total potential difference maintained across the potentiometer wire

can be balanced against a convenient length of the potentiometer wire. The potential difference balanced is proportional to the length of the wire which balances the potential difference.

Home Assignment- Current Electricity

- Define instantaneous current I ? Define 1 Ampere ?

- Write 4 differences between emf and potential difference?

- Define & derive an expression for the drift speed ?

- What do you mean by average relaxation time ?

- Define mobility and find an expression for it in terms of Electric field E ?

- Derive a relation between current and drift velocity ?

- State Ohm’s law with an expression ?

- On what factors the resistance of a conductor depends?

- Define electrical resistivity numerically ?

- What do you mean by current density?

- What do you mean by conductivity?

- Express ohm’s law in microscopic form?

- What do you mean by Temperature coefficient of resistance ?

- Write a note on temperature dependence of resistivity ?

- What are different limitations of Ohm’s Law ?

- Derive an expression for the equivalent resistance in series combination?

- Derive an expression for the equivalent resistance in parallel combination?

- Find the resistance of a resistor having bands as following : four colours are yellow, orange, blue, and gold ?

- Define the terms emf , internal resistance and terminal potential difference?

- Derive an expression between emf and terminal voltage ?

- Write an expression for the equivalent emf when 3 cells are connected in series combination?

- Write an expression for the equivalent emf when 3 cells are connected in parallel combination?

- Write kirchhoff’s two rules ?

- Write the rules/conventions for loop rule of Kirchhoff ?

- Write the principle and working of wheatstone bridge ?

- Write an expression for measuring unknown resistance in meter bridge ?

- Write the principle and working of Potentiometer ?

- Explain how will you use potentiometer to (a) compare emfs of two cells (b) find internal resistance of a cell ?

- How sensitiveness of potentiometer can be increased ?

- What do you mean by Joule’s law of heating ?

- What is power ? Derive an expression for it

- What do you mean by commercial unit of electricity ?

- What do you mean by maximum power theorem ?

NCERT Solved

- (a)  If the electron drift speed is so small, and the electron’s charge is small, how can we still obtain large amounts of current in a conductor?

(b) When electrons drift in a metal from lower to higher potential, does it mean that all the ‘free’ electrons of the metal are moving in the same direction?

(c) The electron drift arises due to the force experienced by electrons in the electric field inside the conductor. But force should cause acceleration. Why then do the electrons acquire a steady average drift speed?

NCERT Unsolved

- The storage battery of a car has an emf of 12 V. If the internal resistance of the battery is 0.4 Ω, what is the maximum current that can be drawn from the battery?

- (a) Three resistors 2 Ω, 4 Ω and 5 Ω are combined in parallel. What is the total resistance of the combination?

(b) If the combination is connected to a battery of emf 20 V and negligible internal resistance, determine the current through each resistor, and the total current drawn from the battery

- Determine the current in each branch of the network shown in Fig. 3.30

- A storage battery of emf 8.0 V and internal resistance 0.5 Ω is being charged by a 120 V dc supply using a series resistor of 15.5 Ω. What is the terminal voltage of the battery during charging? What is the purpose of having a series resistor in the charging circuit?

- Answer the following questions:

  - A steady current flows in a metallic conductor of non-uniform cross-section. Which of these quantities is constant along the conductor: current, current density, electric field, drift speed?

  - Is Ohm’s law universally applicable for all conducting elements? If not, give examples of elements which do not obey Ohm’s law.

  - A low voltage supply from which one needs high currents must have very low internal resistance. Why?

- Determine the equivalent resistance of networks shown in Fig. 3.31

- (a) Given n resistors each of resistance R, how will you combine them to get the (i) maximum (ii) minimum effective resistance? What is the ratio of the maximum to minimum resistance?

- (b) Given the resistances of 1 Ω, 2 Ω, 3 Ω, how will be combine them to get an equivalent resistance of (i) (11/3) Ω (ii) (11/5) Ω, (iii) 6 Ω, (iv) (6/11) Ω?

NCERT Exemplar

- Suppose that a current-carrying ohmic metal wire has a cross-sectional area that gradually becomes smaller from one end of the wire to the other. How do drift velocity, current density,and electric field vary along the wire? Note that the current must have the same value everywhere in the wire so that charge does not accumulate at any one point

- Is the motion of a charge across junction momentum conserving? Why or why not?

- What are the advantages of the null-point method in a Wheatstone bridge?

- What is the advantage of using thick metallic strips to join wires in a potentiometer?

- Why are alloys used for making standard resistance coils?

- In a parallel combination of three resistors which one receives the most power (minimum or maximum) ?

- AB is a potentiometer wire (fig 3.4) . If the value of R is increased,in which direction will the balance point shift?

Objective Questions

1.In the circuit shown(31.3) , the internal resistance  of the cell is negligible. The steady state current in the 2 ohm resistor is: (a) 0.6 A	(b) 0.9 A 	(c) 1.2 A		(d) l.5 A

2. An unknown resistance K1 is connected in series with a resistance of l0 Ω. This combination is connected to one gap of a metre bridge, while a resistance K2 is connected in the other gap. The balance point is at 50 cm. Now when the l0 Ω resistance is removed, the balance point shifts to 40 cm. The value of K1 is: (a)10 Ω  	  (b)20 Ω 		 (c)40 Ω			 (d)60 Ω

3. The temperature of a metal wire rises when an electric current passes through it because:

(a) collision of metal atoms with each other releases heat energy

(b) collision of conduction electrons with each other releases heat energy

(c) when the conduction electrons fall from higher energy level to lower energy level heat energy is released

(d) collision of conduction electrons with the atoms of metal give them energy which appears as heat

4. What is immaterial for an electric fuse?    (a) Its specific resistance                              (b) Its radius

(c) Its length		(d) Current flowing through it			(e) None of the above

5. Constantan wire is used for making standard resistance because it has:

(a) low specific resistance                                                          (b) high specific resistance

(c) negligible temperature coefficient of resistance                     (d) high melting point

6. A potential difference is applied across the ends of a metallic wire. If the potential difference is doubled, the drift velocity: (a) will be doubled (b) will be halved  (c) will be quadrupled     (d) will remain unchanged

7. A fuse is a wire of

(a) low resistance and low melting point.			(b) high resistance  and low melting point .

(c) low resistance  and high melting point .		(d) high resistance and high melting point

8. A wire of resistance R is cut into n equal parts. These parts are then connected in parallel. The equivalent resistance of the combination will be: (a) nR    		(b) R/n                   (c) n/R               (d) R/n2

9.The masses of three wires of copper are in the ratio of 1:3:5 and their lengths are in the ratio 5:3: 1. The ratio of their  electrical resistances is: (a) 1:3:5		(b) 5 :3 :1	(c) 1 : 15:125               (d) 125:15 :1

10. Kirchhoff's first law is based on the law of conservation of:

(a) charge              (b) energy             (c) momentum                                         (d) sum of mass and energy

11. Kirchhoff's second law is based on the law of conservation of:

(a) charge             (b) energy             (c) momentum                                         (d) sum of mass and energy

(c) momentum

12. In an electrolyte 3.2 x 1018 bivalent positive ions drift to the right per second while 3.6 x l018 monovalent negative ions drift to the left per second. Then, the current is:

(a) 1.6 amp to the left       (b) 1.6 amp to the right      (c) 0.45 amp to the left           (d) 0.45 amp to the right

13. In the following figure(31.4), the reading of an ideal voltmeter V is zero. Then, the relation between R, r1 and r2 is: (a) R =r2 -  r1             (b) R = r1 -  r2                   (c) R = r1 + r2                   (d) R = r1 r2 /(r1 + r2)

14. A circuit whose resistance R is connected to n similar cells. If the current in the circuit is the same whether the cells are connected in series or in parallel, then the internal resistance

r of each cell is given by: (a) R/n                  (b) nR 		(c) R              		 (d) 1/R

15.On increasing the temperature of a conductor, its resistance increases because the :

(a) relaxation time increases      (b) mass of electron increases               (c) electron density decreases

(d) relaxation time decreases       (e) relaxation time remains constant

16.Find the true statement.

(a) Ohm's law is applicable to all conductors of electricity.

(b) In an electrolyte solution, the electric current is mainly due to the movement of electrons.

(c) The resistance of an incandescent lamp is lesser when the lamp is switched on.

(d) Specific resistance of a wire depends upon its dimension.

(e) The resistance of carbon decreases with the increase of  temperature

17. The wire of the electric fire element glows red hot but the copper wire in the leads from the plug remains cool, although the same current is passing through each. This is because the element wire:

(a) is longer than the copper wire              (b) has much higher resistance than the copper wire

(c) is coiled but the copper wire is not       (d) has a much lower resistance than the copper wire

18. A current flows in a wire of circular cross-section with the free electrons travelling with a mean drift velocity v. If an equal current flows in a wire of twice the radius, new mean drift velocity is

(a) v                  (b) v /2              		(c) v/4                           		 (d) none of these

19.The resultant resistance value of n resistances, each of r ohm when connected in parallel, is x. When these n resistances are connected in series, the resultant value is:

(a) nx                    (b) n2x		(c)  r n x		 (d) r2x/n

20. If a wire is stretched to make it 0.1% longer, its resistance   will:

(a) increase by 0.05%	 (b) increase by 0.2%            (c) decrease by 0.2%     (d) decrease by 0.05%

21. An electron is moving in a circular path of radius 0.5m with a period of 10-19 s. The current constituted in the circular path is : (a) 0.1 A                  (b) 2.5A                 (c) 1.9 A               (d) 1.6 A

22. A wire of resistance R is stretched till its radius is half of the original value. Then, the resistance of the stretched wire is     (a) 2R	(b) 4R			(c) 8R			(d)16R

23. A wire of resistance R is stretched till its length is double of the original wire. Then, the resistance of the stretched wire is: (a) 2R			(b) 4R			(c) 8R			(d)16R

24. The temperature of a metal wire rises when an electric current passes through it because:

(a) collision of metal atoms with each other releases heat energy

(b) collision of conduction electrons with each other releases heat energy

(c) when the conduction electrons fall from higher energy level to lower energy level heat energy is released

(d) collision of conduction electrons with the atoms of metal give them energy which appears as heat

25. For a metallic wire, the ratio V/I

(a) increases or decreases as the temperature rises, depending upon the metal

(b) decreases as the temperature rises                              (c)  independent of temperature

(d) increases as the temperature rises

26. A wire of length L is drawn such that its diameter is reduced to half of its original diameter. If the initial resistance of the wire were 10Ω, its new resistance would be :

(a) 40Ω                  (b) 80 Ω                         (c) 120 Ω                               (d) 160Ω

27. The dimensions of a block are 1 cm x 1 cm x 100 cm. If the specific resistance of its material is 3 x 10-7 ohm-metre, then the resistance between the opposite rectangular faces is

(a)3 x 10-9 ohm 		(b)3 x 10-7 ohm			(c)3 x 10-5 ohm			(d)3 x 10-3 ohm

28. The dimensions of a block are 1 cm x 1 cm x 100 cm. If the specific resistance of its material is 3 x 10-7 ohm-metre, then the resistance between the square faces is

(a)3 x 10-9 ohm 		(b)3 x 10-7 ohm			(c)3 x 10-5 ohm			(d)3 x 10-3 ohm

29. A cell of emf E and internal resistance r is connected in series with an external resistance nr. Then, the ratio of the terminal potential difference to emf is: (a) (1/n)         (b) 1/(n +1)          (c) n/(n+1)          (d) (n +1)/n

30. Four wires of equal length and of resistance 5 ohm each are connected in the form of a square. The equivalent resistance between the diagonally opposite corners of the square is:

(a) 5 ohm	(b) 10 ohm	(c) 20 ohm            (d) 5/4 ohm

31.A potentiometer wire, 10 m long, has a resistance of 40 Ω. It is connected in series with a resistance box and a 2V storage cell. If the potential gradient along the wire is (0.1 mV/cm),the resistance unplugged in the box is: (a) 260 Ω          (b) 760 Ω                (c) 960 Ω                  (d) 1060 Ω

32.In a closed circuit, the current I (in ampere) at an instant of time t (in second) is given by: I = 4 - 0.08t. The number of electrons flowing in 50 s through the cross-section of the conductor is:

(a) 1.25 x 1019		(b) 6.25	x 1020	          (c) 5.25 x 1019		(d) 2.25 x 1020        (e) 4.25 x 1020

33.The resistance of a wire at room temperature 300C is found to be 10Ω. Now to increase the resistance by 10%, the temperature of the wire must be: [The temperature coefficient of resistance of the material of the wire is  0.002/0C]    (a) 360C                (b) 830C                  (c) 630C                (d) 330C                     (e) 660C

34.The value of current I in the following circuit(31.9) is:

(a) 3 A		 (b) 13 A		 (c) 23 A 		(d) -3 A

35.Assume that each atom of copper contributes one electron. If the current flowing through a copper wire of 1mm diameter is 1.1A, the drift velocity of electrons will be:(Density of Cu = 9 gmcm-3, at. wt. of Cu = 63)

(a) 0.3 mm/s               (b) 0.5 mm/s                       (c) 0.1 mm/s                   (d) 0.2 mm/s

36. There is a current of 1.344 amp in a copper wire whose area of cross-section normal to the length of the wire is 1 mm2. If the number of free electrons per cm3 is 8.4 x1022, then the drift velocity would be:

(a) 1 mm/s               (b) 1 m/s                       (c) 0.1 mm/s                   (d) 0.01 mm/s

37. When a metal conductor connected to left gap of a metre bridge is heated, the balancing point:

(a) shifts towards right                              (b) shifts towards left

(c) remains unchanged                             (d) remains at zero

38. The temperature coefficient of resistance of a wire is 0.001250C-1. At 300 K its resistance is one ohm. The

resistance of the wire will be 2 ohm at:   (a) 1154 K             (b) 1100 K                (c) 1400 K           (d) 1127 K

39.A wire P has resistance of 20 ohm. Another wire Q of same material but length twice that of P has resistance of 8 ohm. If r is the radius of cross-section of P, the radius of cross-section of Q is:

(a) r                     (b) r/2		(c) r5                        (d) 2r

40. In the following figure(31.11), the value of resistor to be connected between C and D so that the resistance of the entire circuit between A and B does not change with the number of elementary sets used is:

(a) R         (b) R(3-1)           (c) 3R                (d) R(3 + 1)

41.lf the electric current through an electric bulb is 3.2 A, the number of electrons flowing through it in 1 second is: (a) 2 x 109		(b) 2 x 1019		(c) 3.2 x 109		(d) 1.6 x 1018

42.A current of 6 A enters one corner P of an equilateral triangle PQR(31.57) having 3 wires of resistances 2Ω each and leaves by the corner R. Then, the current I1 and I2 are(a) 2A, 4 A     (b) 4 A, 2 A   (c)  l A, 2A      (d)  2A , 3 A

43. 206. Two resistors of resistances 200 kΩ and 1 MΩ respectively form a potential divider with outer junctions maintained at potentials of + 3 V and -15 V as shown in fig 31.118. Then, the potential at the junction between the resistors is:

(a) + 1 V              (b) -0.6 V                    (c) zero                        (d) -12 V                (e) + l2V

44.At room temperature, copper has free electron density of  8.4 x 1028 m3.The electron drift velocity in a copper conductor of cross-sectional area of 10-6 m2 and carrying a current of 5.4 A, will be:

(a) 4m/s                (b) 0.4 m/s             (c) 4 cm/s             (d) 0.4 mm/s

45.In a metre bridge experiment, if the ratio of the left gap resistance to right gap resistance is 2 :3, the balance point from left is at: (a) 60 cm            (b) 50 cm                  (c) 40 cm              (d) 20 cm

46.Three resistances P, Q, R each of 2 Ω and an unknown resistance S form the four arms of a Wheatstone bridge circuit. when a resistance of 6 Ω is connected in parallel to S the bridge gets balanced. What is the value of S ?             (a)3 Ω	 (b)6 Ω		 (c)1 Ω  		(d)2Ω

47.An electric cable of copper has just one wire of radius 9 mm. Its resistance is 5 ohm. This single copper wire of the cable is replaced by 6 different well insulated copper wires each of radius 3mm.The total resistance of the cable will now be equal to (a) 7.5 ohm	(b) 45 ohm	(c) 90 ohm            (d) 270 ohm

48.There is a current of 40 amp in a wire of 10-6 square metre area of cross-section. If the number of free electrons per cubic metre is 1029, then the drift velocity is:

(a) 250 x 10-3 m/sec   (b) 25.0 x l0-3 m/sec        (c) 2.50 x l0-3 m/sec           (d) 1.25 x 103 m/sec

49. A battery of 6 volt is connected to the terminals of a three-metre long wire of uniform thickness and resistance of the order of 100 ohm. The difference of potential between two points separated by 50 cm on the wire will be:  (a) 1 volt                  (b) 1.5 volt                    (c) 2 volt                          (d) 3 volt

50. An energy source will supply a constant current into the load if its internal resistance is

(a) zero		                                        (b) non zero but less than the resistance of the load

(c) equal to resistance of the load              (d) very large as compared to the load resistance

51.A battery of emf E has an internal resistance r. A variable resistance R is connected to the terminals of the battery. A current I is drawn from the battery. V is the terminal potential difference. If R alone is gradually reduced to zero, which of the following best describes I and V ?

(a) I approaches zero, V  approaches  E ; (b) I approaches E/r , V  approaches  zero ;

(c) I approaches E/r, V  approaches  E ; (a) I approaches , V  approaches  E ;

52.A battery has an emf of 15 V and internal resistance of 1Ω .Is the terminal to terminal potential difference less than, equal to or greater than 15 V if the current in the battery is: (i) from negative to positive terminal, (ii) from positive to negative terminal (iii) zero current?

(a) less, Greater, Equal    (b) Less, Less, Equal       (c) Greater, Greater, Equal       (d) Greater, Less, Equal

53.Twelve cells, each having emf 'E' volts are connected in series and are kept in a closed box. Some of these cells are wrongly connected with positive and negative terminals reversed. This 12 cell battery is connected in series with an ammeter, an external resistance 'R' ohm and a two-cell battery (two cells of the same type used earlier, connected perfectly in series). The current in the circuit when the 12-cell battery and 2-cell battery aid each other is 3A , and is 2A when they oppose each other. Then, the number of cells in l2-cell battery that are connected wrongly is: (a) 4      (b) 3           (c) 2                  (d) 1

54. To get maximum current through a resistance of 2.5 Ω, one can use m rows of cells, each row having n cells. The internal resistance of each cell is 0.5 Ω. What are the values of n and m if the total number of cells is 45: (a) m=3,n=15		(b) m=5   , n=9		(c) m=9,n=5			(d) m=15,n=3

55.One end each of a resistance 'r' ,capacitance C and resistance '2r' are connected together (31.63). The other ends are respectively connected to the positive terminals of the batteries P, Q, R having respectively emfs E, E and 2E. The negative terminals of the batteries are then connected together. In this circuit, with steady current the potential drop across the capacitance is:     (a) E/3        (b) E/2           (c) 2E/3        (d) E

56.Two flash light electric incandescent lamps, each requiring 3 amp at 1.5 V are placed in series and connected to a 6 V cell. What resistance must be connected in series to operate them?

(a) 22 Ω             (b) 2 Ω         (c) 6 Ω                    (d) 1 Ω

57. The number of dry cells, each of emf 1.5 volt and internal resistance 0.5 Ω that must be joined in series with a resistance of 20 ohm so as to send a current of 0.6 ampere through the circuit is:

(a) 2		 (b) 8			 (c) 10 			(d) 12

58.A teacher asked a student to connect 'N' cells each of emf 'e' in series to get a total emf of Ne. While connecting, the student, by mistake, reversed the polarity of 'n' cells. The total emf of the resulting series combination is:     (a)  e (N - n/2)              (b) e (N - n)               (c) e (N - 2n)              (d) eN

59. Resistance of rod is l Ω. It is bent in the form of square. What is the resistance across ad joint corners?

(a) l Ω                        (b) 3 Ω                        (c) (3/16)Ω                                      (d) (3/4) Ω

60. A hollow copper tube of 5 metre length has got external diameter equal to 10 cm and its walls are 5 mm thick. The specific resistance of copper is 1.7 x 10-8 ohm- metre. The resistance of the copper tube approximately is:(a) 5.6 x 10-3 Ω          (b) 5.6 x 10-9 Ω          (c) 5.6 x 10-5 Ω          (d) 5.6 x 10-7 Ω

61. Two cells of emfs E1 and E2 and internal resistances r1 and r2 are connected in parallel. Then, the emf and internal resistance of the equivalent source is:

62. A galvanometer has a resistance of 3663 ohm. A shunt S is connected across it such that (1/34) of the total current passes through the galvanometer. Then, the value of shunt is:

(a) 3663 Ω             (b) 111 Ω         (c) 107.7 Ω                    (d) 3555.3 Ω

63. A galvanometer has a resistance of 3663 ohm. A shunt S is connected across it such that (1/34) of the total current passes through the galvanometer. Then, the combined resistance of the shunt and the galvanometer is: (a) 3663 Ω             (b) 111 Ω         (c) 107.7 Ω                    (d) 3555.3 Ω

64. A galvanometer has a resistance of 3663 ohm. A shunt S is connected across it such that (1/34) of the total current passes through the galvanometer. Then the external resistance which must be connected in series with the main circuit, so that the total current in the main circuit remain unaltered even when the galvanometer is shunted, is: (a) 3663 Ω             (b) 111 Ω         (c) 107.7 Ω                    (d) 3555.3 Ω

65. A galvanometer has a resistance of 3663 ohm. A shunt S is connected across it such that (1/34) of the total current passes through the galvanometer. The multiplying power of the shunt is:

(a)1		 (b) 33 			(c) 34			 (d) 35

66. A galvanometer has a resistance of 3663 ohm. when a shunt of 111 Ω is applied across the galvanometer, its sensitivity becomes: (a) 33 fold	 (b) (1/33) fold 		(c) 34 fold		 (d) (1/34) fold

67.Thermal speed of free electrons is of the order of:(a) zero    (b) 10-2 cm/sec      (c) 105 m/sec  (d) 1 m/sec

68.Drift speed of electrons is of the order of: (a) zero    (b) 10-2 cm/sec      (c) 105 m/sec  (d) 1 m/sec

69.The potential gradient along the length of a uniform wire is 10 volt per metre. B and C are two points at 30 cm and 60 cm on a metre scale along the wire. The potential difference between B and C will be:

(a) 3 volt (b) 0.4 volt (c) 7 volt (d) 4 volt

70. The electron drift speed is small and the charge of the electron is also small but still, we obtain a large current in a conductor. This is due to:

(a) the conducting property of the conductor		              (b) the resistance of the conductor is small

(c) the electron number density of the conductor of is small

(d) the electron number density of the conductor is enormous.

71.Two copper wires of length L and 2L have radii, r and 2r respectively. What is the ratio of their specific resistances?   (a)1:2       (b)2:1          (c) 1: 1        (d)1:3

72.A circuit consists of five identical conductors as shown in the adjoining figure (31.62). The two similar conductors are added as indicated by the dotted lines. The ratio of resistances before and after addition will be:

(a) (7/5) 		(b) (3/5)		(c) (5/3) 		(d) (6/5)

73.A block has dimensions 1 cm, 2 cm and 3 cm. Ratio of the maximum resistance to minimum resistance between any pair of opposite faces of this block is  (a)9:l        (b)1:9          (c) 18: 1        (d)1:6

74. In the circuit shown in the following figure(31.20), the potential difference between points A and B will be: (a) (8/9) volt	(b) (4/3) volt	(c) (2/3) volt       (d) 2 volt

75. In the adjoining figure(31.21) the emf of the cell is 2 V and internal resistance is negligible. The resistance of the voltmeter is 80 ohm. The reading of voltmeter will be:

(a) 2.00 volt            (b) 1.33 volt             (c) 1.60 volt            (d) 0.80 volt

76. Pieces of aluminium (Al) and germanium (Ge) are cooled from T1 K to T2 K The resistance of:(a) each of them increases               (b) each of them decreases

(c) aluminium increases and that of germanium decreases

(d) aluminium decreases and that of germanium increases

77.  The colour code for a resistor of resistance 3.5 kΩ with 5% tolerance is

(a) orange, green, orange and gold			(b) red, yellow, black and gold

(c) orange, green, orange and silver	(d) orange, green, red and silver      (e) orange, green, red and gold

78. A battery of internal resistance one ohm and emf 3 volt sends a current through 1 metre of uniform wire of resistance 5 Ω. The poles of the cell of emf 1.4 volt are connected to two points on the wire so that no current passes through this cell. Then, the potential gradient of the wire is:

(a) 2.5 volt 		(b) 2.5 volt/metre 		(c) 3 volt/metre 			(d) 1.5 volt/metre

79.Calculate current i in the adjoining circuit (31.61).

(a) 0.1 amp          (b) 0.2 amp           (c) 0.4 amp           (d) 0.3 amp

80. The resistivity of a potentiometer wire is 40 x l0-8 ohm-m and its area of cross-section is 8 x 10-6 m2. If 0.2 ampere current is flowing through the wire, the potential gradient will be:

(a) 10-2 v/m 		(b) 10-1 v/m		(c) 3.2 x 10-2 v/m			(d) 1 v/m

81.For a cell of  emf 2V a balance is obtained for 50 cm of the potentiometer wire. If the cell is shunted by a 2 Ω resistor and the balance is obtained across 40 cm of the wire, then the internal resistance of the cell is:

(a) 0.25 Ω		 (b) 0.50 Ω		 (c) 0.80 Ω		 (d) 1.00 Ω

82. A uniform wire of resistance 20 ohm having resistance 1 Ω /m is bent in the adjoining form of a circle as shown in the figure(31.23). If the equivalent resistance between M and N is 1.8 Ω ,then the length of the shorter section is: (a) 2 m	     (b)5m	(c) 1.8 m              (d) 18 m

83. Total number of electrons present in 4 amp current flowing for 1 sec is:

(a) 2.5  x l019            (b) 2.5  x l020            (c) 1.25  x l013            (d) 6  x l08

84. In the following circuit diagram(31.25) the potential difference across the plates of the capacitor C is: a) 2.5 volt       (b) 1.5 volt      (c) 1.0 volt       (d) zero

85.An electron is circulating in a circular path with a frequency of 50 Hz. What is the associated current?(a) 0.8 x l0-7 A            (b)8  x  l0-17 A            (c) 0.4 x l0-17 A               (d) 80 x l0-17 A

86.In the adjoining circuit (31.60), the potential difference across 3 Ω is:

(a) 2 V           (b)4 V              (c)8V                   (d) 16 V

87.The ratio of the resistance of conductor at temperature 150C to its resistance at temperature 37.50C is 4 : 5. The temperature coefficient of resistance of the conductor is:

(a) 1/25 0C-1	(b) 1/50 0C-1	   (c) 1/80 0C-1 			(d) 1/75 0C-1 		     (e) 1/40 0C-1

88.The plot (31.59) represents the flow of current through a wire at three different times. The ratio of charges flowing through the wire-at different times is:

(a)2:l:2		(b)l:3:3		(c)1:1:1		(d) 2:3 : 4		(e)2:3:3

89.32 cells, each of emf 3 V, are connected in series and kept in a box. Externally, the combination shows an emf of 84 V. The number of cells reversed in the combination is:

(a) zero		 (b) 2 			(c) 4 			(d) 8 		     (e) 6

90.Two identical cells whether connected in parallel or in series give the same current when connected to an external  resistance of 1.5 Ω. Find the value of internal resistance of each cell

(a) 1 Ω              (b) 0.5 Ω             (c) zero                  (d) 2 Ω                 (e) 1.5 Ω

91.The resistance of a bulb filaments is 100 Ω at a temperature of 1000C. If its temperature coefficient of resistance be 0.005/0C, its resistance will become 200 Ω at a temperature of: (a) 2000 C (b) 3000 C (c) 4000 C (d) 5000 C

92.The current I drawn from the 5 volt source (31.58) will be:

(a) 0.17 A      (b) 0.33 A         (c) 0.5 A            (d) 0.67 A

93.In a Wheatstone bridge, three resistance P, Q and R are connected in the three arms and fourth arm is formed by two resistances S1 and S2 connected in parallel. The condition for bridge to be balanced will be:

(a) P/Q  = R/(S1+S2) 	(b) P/Q  = 2R/(S1+S2)	(c) P/Q  = R(S1+S2) /S1S2	 (d) P/Q  = R(S1+S2) /2S1S2

94.A material B has twice the specific resistance of A. A circular wire made of B has twice the diameter of a wire made of A. Then, for the two wires to have the same resistance, the ratio LB/LA of their respective lengths must be: (a) 2 		(b) 1		(c) ½		 (d) ¼

95.A conducting wire is stretched to increase its length by 2%. What is the percentage change in its resistance? (a) 2%		 (b) 4%			 (c) 8% 			(d) l%

96.You are given n resistors, each of resistance r. They are first combined to get minimum possible resistance, then they are connected to get maximum possible resistance. The ratio between minimum to maximum resistance is: a) l/n              (b) n                   (c) n2                      (d) 1/n2

97. when the key K is pressed(31.30) at time t=0  then which of the

following statements about the current I in the resistor AB of the given circuit is true?  (a) I = 1 mA at all times  (b) I = 2 mA at all times

(c) I oscillates between I mA and 2 mA

(d) At t = 0,I = 2 mA and with time it goes to1 mA

98.The balancing length for a cell is 560 cm in a potentiometer  experiment. When an external resistance of l0 Ω is connected in parallel to the cell, the balancing length changes by 60 cm. The internal resistance of the cell (in ohm) is: (a) 1.6 		(b) 1.4			 (c) 1.2			 (d) 0.12

99. A conductor of resistance 3 Q is stretched uniformly till its length is doubled. The wire is now bent in the form of an equilateral triangle. The effective resistance between the ends of any side of the triangle (in ohm) is: (a) 9/2 		(b) 8/3			 (c) 2			 (d) 1

100. A uniform conductor of resistance R is cut into 20 equal pieces. Half of them are joined in series and the remaining half of them are connected in parallel. If the two combinations are joined in series, the effective resistance of all the pieces is: (a) R		(b) R/2              (c) l0l R/200               (d) 201R/200

101. when a current flows in a wire, there exists an electric field in the direction of:

(a) flow of current                                        (b) opposite to the flow of current

(c) perpendicular to the flow of current        (d) at an angle of 450 to the flow of current

102. The drift velocity of free electrons in a conductor is v when a current i is flowing in it. If both the radius and current are doubled, then drift velocity will be: (a) v		(b) v /2              (c) v /4          (d) v /8

103.A piece of copper and another of germanium are cooled from room temperature to 40 K. The resistance: (a) each of them increases(b) copper increases and germanium decreases

(c) copper decreases and germanium increases (d) each of them decreases

104. The sides of a rectangular block are 2 cm, 3 cm and 4 cm. The ratio of maximum to minimum resistance between its parallel faces is: (a) 4 		(b) 3		 (c) 2		 (d) 1

105. Three equal resistances each of 3 Q are in series and connected to a cell of internal resistance one ohm. If these resistances are in parallel and connected to the same cell, then the ratio of the respective currents through the electric circuits in the two cases is: (a) 1/8     	(b) 1/7		 (c) 1/5		 (d) 1/3

106. Two different conductors have same resistance at 0"C. It is found that the resistance of the first conductor at t10C, is equal to the resistance of the Second conductor at t20C. The ratio of the temperature coefficients of resistance of the conductors, 1/2 is

(a) t1/ t2    	(b) (t2 - t1)/ t2    		 (c) (t2 - t1)/ t1    		 (d) t2/ t1             (e) t2/ (t2 - t1)

Answers Explanations Objective Questions- Current Electricity

| - B | - B | - D | - E |
| --- | --- | --- | --- |
| - C | - A | - B | - D |
| - D | - A | - B | - B |
| - B | - C | - D | - E |
| - B | - C | - B | - B |
| - D | - D | - B | - D |
| - D | - D | - B | - D |
| - C | - A | - B | - B |
| - B | - B | - C | - C |
| - A | - D | - C | - B |
| - B | - A | - C | - D |
| - C | - A | - A | - C |
| - A | - A | - B | - D |
| - D | - A | - A | - D |
| - C | - C | - C | - C |
| - C | - B | - C | - D |
| - C | - D | - C | - B |
| - A | - D | - C | - C |
| - A | - C | - B | - D |
| - E | - B | - D | - A |
| - B | - A | - A | - D |
| - A | - A | - D | - C |
| - B | - E | - B | - C |
| - C | - A | - B | - D |
| - D | - C | - B | - C |
| - A | - B | - C | - A |
| - C | - D |  |  |